# Conveyor Belt Splice Failure Investigation Package

## Section 1 – Executive Summary

On 04-Jun-2026, Conveyor CV-03 experienced an unexpected shutdown following activation of the belt misalignment switch. Initial inspection revealed a significant opening at the mechanical splice located approximately 18 meters downstream of the loading zone.

The conveyor is part of the primary material handling system and transports crushed limestone from the primary crusher to the screening area. Production loss associated with the incident was estimated at 7.5 hours.

This package compiles all available information prior to the formal RCA process.

# Section 2 – Asset Information

Asset Tag: CV-03

Manufacturer: Continental Belt Systems

Installation Date: March 2025

Belt Width: 1200 mm

Belt Length: 187 m

Operating Speed: 2.5 m/s

Rated Capacity: 900 tph

Material: Crushed Limestone

Maximum Lump Size: 180 mm

Operating Schedule: 24/7

# Section 3 – Failure Description

At approximately 13:42, operators reported a repetitive impact noise occurring once per belt revolution.

At approximately 14:15, tracking drift was observed toward the drive-side frame.

At approximately 16:02, the belt misalignment switch activated.

Maintenance personnel discovered a longitudinal opening in the splice region.

The opening measured approximately 210 mm in length.

Material accumulation was present around the damaged section.

Signs of abrasion and discoloration were visible near the failure zone.

# Section 4 – Production Impact

Production Loss: 6,750 tons

Downtime: 7.5 hours

Estimated Financial Impact: $84,000

Delayed Shipments: 3

Recovery Time: 11 hours

Emergency Labor Hours: 42

# Section 5 – Historical Failures

### Failure Record 2023-17

Splice edge separation.

Temporary repair completed.

Root cause not investigated.

### Failure Record 2024-08

Belt mistracking.

Multiple tension adjustments performed.

No permanent solution documented.

### Failure Record 2024-19

Pulley lagging wear.

Increased vibration reported.

Corrective action delayed due to production requirements.

### Failure Record 2025-04

Material buildup under return idlers.

Cleaning conducted.

Issue recurred after three weeks.

# Section 6 – Maintenance Work Orders

WO-11234

Description:
Adjust conveyor tension.

Reason:
Tracking issue.

Technician Notes:
Belt appears slightly elongated.

WO-11387

Description:
Inspect splice condition.

Result:
No major abnormalities observed.

Recommendation:
Continue monitoring.

WO-11802

Description:
Emergency splice repair.

Details:
Minor edge delamination repaired.

Estimated downtime:
1.5 hours.

WO-11992

Description:
Replace damaged return idler.

Observation:
Excessive wear noted.

# Section 7 – Shift Handover Notes

March 10

"Conveyor sounds rough near drive section."

March 18

"Tracking correction required twice."

April 4

"Small amount of material leaking near splice."

April 27

"Visible belt oscillation under heavy load."

May 9

"Tracking appears unstable after rain."

May 28

"Operator reports clicking noise every rotation."

June 2

"Recommend detailed splice inspection."

No action documented.

# Section 8 – Operator Statements

Operator A:

"I first noticed the belt wandering about two weeks ago."

Operator B:

"There was a bump whenever the splice passed over the pulley."

Operator C:

"The noise got worse during higher production periods."

Operator D:

"The splice looked thicker than surrounding belt sections."

Operator E:

"Sometimes material would bounce near the loading zone."

# Section 9 – Supervisor Interview

Production supervisor indicated that throughput targets increased significantly during May.

Several requests for maintenance downtime were postponed.

Conveyor remained operational despite repeated tracking complaints.

Supervisor believed issue was minor because no alarms were generated.

# Section 10 – Reliability Engineer Notes

Repeated tension adjustments suggest progressive belt elongation.

Historical records indicate recurring splice-related concerns.

Potential fatigue mechanism should be investigated.

Need verification of installation quality.

Need review of dynamic loading conditions.

# Section 11 – Environmental Conditions

Average Ambient Temperature:
38°C

Maximum Recorded Temperature:
44°C

Rainfall Events:
11

Dust Level:
High

Humidity:
Variable

Environmental contamination around return side observed.

# Section 12 – Belt Installation History

Original installation performed by contractor.

No formal splice audit available.

Acceptance inspection checklist incomplete.

Torque records missing.

Quality verification records unavailable.

# Section 13 – Inspection Records

Inspection 1

Date: March 15

Result:
Acceptable.

Inspection 2

Date: April 17

Result:
Minor wear.

Inspection 3

Date: May 29

Result:
Tracking variation noted.

Recommendation:
Follow-up inspection.

No follow-up found.

# Section 14 – Spare Parts Records

Three emergency splice kits used in last 18 months.

Higher consumption compared with similar conveyors.

No investigation documented.

# Section 15 – Material Characteristics

Material occasionally contains oversized rocks.

Moisture content varies seasonally.

Impact loading spikes reported.

Segregation observed during wet conditions.

# Section 16 – Witness Statements

Maintenance Technician #1

"The splice never looked completely flat."

Maintenance Technician #2

"I suspected tension was too high."

Maintenance Technician #3

"I thought alignment was contributing."

Maintenance Technician #4

"We were planning inspection next shutdown."

# Section 17 – Preliminary Mechanical Findings

Possible localized stress concentration.

Possible fatigue crack propagation.

Potential splice installation deficiency.

Potential pulley misalignment.

Potential impact damage.

Potential contamination ingress.

# Section 18 – Potential Failure Modes

- Fatigue failure.

- Over-tension failure.

- Splice manufacturing defect.

- Installation error.

- Misalignment induced damage.

- Impact damage.

- Material ingress.

- Corrosion of reinforcement.

- Progressive delamination.

- Combined mechanism.

# Section 19 – Timeline of Events

(Multi-page chronological log from installation through failure.)

Month-by-month operating history.

Maintenance interventions.

Inspection activities.

Production changes.

Environmental events.

Operator complaints.

Management decisions.

# Section 20 – Open Questions

Why were repeated tracking issues occurring?

Why was splice repair required previously?

Why were recommendations not acted upon?

Was production increase a contributing factor?

Did maintenance practices accelerate damage?

Was original installation compliant?

# Section 21 – Data Required

Belt tension measurements.

Alignment survey.

Splice installation procedure.

Contractor records.

OEM recommendations.

Material loading profile.

Motor current trends.

Vibration history.

Thermal imaging records.

Downtime history.

# Section 22 – Hypothesis Log

Hypothesis A:
Improper splice installation.

Hypothesis B:
Progressive fatigue.

Hypothesis C:
Overload events.

Hypothesis D:
Pulley alignment deviation.

Hypothesis E:
Combination of factors.

Status:
Unconfirmed.

# Section 23 – Evidence Matrix

Evidence ID 001–100

Source

Description

Confidence

Verification Status

Related Failure Mode

Corrective Action Link

# Section 24 – Risk Assessment

Probability of recurrence: High

Operational impact: High

Safety impact: Medium

Environmental impact: Low

Business impact: High

# Section 25 – Lessons from Similar Plants

Plant A:
Splice failure due to excessive tension.

Plant B:
Splice failure due to mistracking.

Plant C:
Fatigue failure from repeated overload events.

Plant D:
Contractor installation deficiency.

# Section 26 – Engineering Assessment

Review of dynamic loading.

Review of belt design margin.

Review of splice selection.

Review of operating envelope.

Review of maintenance strategy.

# Section 27 – Corrective Action Candidates

Alignment verification.

Splice redesign.

Inspection frequency increase.

Condition monitoring installation.

Training improvement.

Loading control.

Contractor qualification review.

# Section 28 – Long-Term Reliability Recommendations

Implement predictive inspections.

Introduce splice health monitoring.

Improve maintenance planning.

Establish RCA trigger criteria.

Create inspection standards.

Track recurring defects.

# Section 29 – Appendices

Photographs.

Work orders.

Inspection forms.

Operator logs.

Maintenance history.

Production reports.

OEM documents.

# Section 30 – Investigation Readiness Summary

The available evidence suggests that the failure likely developed progressively over several months. Multiple warning signs existed prior to the incident, including tracking instability, unusual noise, previous splice repairs, and increasing maintenance interventions.

At this stage no single root cause can be confirmed. Additional inspection, material analysis, maintenance review, and operating condition assessment are required before causal factors can be validated.